Life Cycle Assessment for Anaerobic Digestion Process Treating Source Separated Food Waste Generated in Local Municipalities in Sri Lanka.

B. K. T. Samarasiri1,*, P. G. Rathnasiri1, S. H. P. Gunawardena1

1 Department of Chemical and Process Engineering, University of Moratuwa, Moratuwa, Sri Lanka

* Tel: +94 71 1402908, E-mail: mr.kasun.samarasiri@ieee.org

The short term biodegradable food waste is the major fraction in municipal solid waste generated in local authorities in Sri Lanka. Currently waste management practices such as open dumping used to manage this food waste more commonly and in a very few local authorities waste to energy via composting and anaerobic digestion are carried out. The anaerobic digestion has the potential to overcome the current issues related with improper management of food waste while generating the electricity and organic fertilizers for agricultural uses. In order to analyse the socio-economic and environmental impact on anaerobic digestion of food waste, a life cycle assessment (LCA) model has been developed and applied to a large scale local open dumping site in Sri Lanka. The selected food waste slurry for the analysis contained TS 70g/l, VS 55.6g/l, 3% fat, 13% proteins and 58.5% lipids. While operating temperature was 35°C and VS 55.6g/l, the best performance of the anaerobic reactor was shown at the hydraulic retention time (HRT) of 11.9 days. According to the model, at this HRT, the methane yield was 0.25 m³/kgVSS added, VS reduction efficiency was 50% and power generation via food waste was 669.66kJ/kg. The socio-economic and environmental feasibility of food waste arrival at different local authorities can be addressed by using this LCA model.

.

Keywords: Life cycle assessment, Municipal solid waste, Anaerobic digestion.